Lab  Photo-interrupt

Introduction

A photo-interrupter is a sensor that arranges light-emitting component and light-receiving

component face-to-face and packages them together. It applies the principle that light is

interrupted when an object passes through the sensor. Therefore, photo-interrupters are

widely used in speed measurement.

|  | Components - 1 * Arduino Uno board - 1 * USB data cable - 1 * Photo-interrupter module - Several jumper wires |
| --- | --- |

Experimental Principle

Basically a photo-interrupter consists of two parts: transmitter and receiver. The transmitter

(e.g., an LED or a laser) emits light and then the light goes to the receiver. If that light beam

between the transmitter and receiver is interrupted by an obstacle, the receiver will detect

no incoming light even for a moment and the output level will change. In this experiment,

we will turn an LED on or off by using this change.

Experimental Procedures

Step 1: Build the circuit

Photo-interrupter Module          Arduino Uno

S --------------------------------------- Pin7

GND -------------------------------------- GND

Vcc --------------------------------------- 5V

Step 2: Program

Step 3: Compile

Step 4: Upload the sketch to Arduino Uno

Task: stick a piece of paper in the gap of the sensor, and the LED attached to pin 13 on the

Arduino Uno will go out; remove the paper, and then the LED attached to pin 13 will light up again.

A) Design and implement

Hardware implementation with the Arduino board and the bread board

B) Program

const int photoPin = 7; //the number of the key pin

const int ledPin = 13;//the number of the led pin /***************************************************/

void setup()

{

pinMode(photoPin,INPUT);

pinMode(ledPin,OUTPUT);//initialize the led pin as output

pinMode(ledPin,OUTPUT); //initialize led as an output

}

/***************************************************/

void loop()

{

boolean Value=digitalRead(photoPin);

//if it is blocked, the state is HIGH, turn off LED

if(Value ==HIGH )

{

digitalWrite(ledPin,LOW);//turn off the led

}

else

{

digitalWrite(ledPin,HIGH);//turn on the led

}

}

/******************************************************/

C) Test

- Test theHardware implementation with the arduino board

- Change the program or connection for the LEDs as below

Original program	your program to be modified

Stick a piece of paper		LEDs: Off		LEDs: On
Remove the paper		LEDs : On		LEDs : Off	
Signal				pin 7			pin 8

Deliverables

Lab report

    - Introduction

    - Body: How to connect, explanation of the sensor internal circuit, debugging, and others

    - Conclusion : lessons from the experiment

    - Attachments: revised code, picture